ECE 465 Lab 2

Boolean Minimization and Prime Implicants

- Introduction

When creating circuits, there are some Boolean algebra expressions that  will create large circuits, that at the end of the day, are redundant. Take the expression:

ABC+A'+A'BC

The way this is expressed is pictured below in the circuit. Note that we are not using more than 2-input and/or gates:

This circuit costs 3-And gates and 2-or gates. Using basic algebra minimization, we can get the same logic circuit for less.

ABC+A'+A'BC – We can apply the absorption theorem

ABC+A' - Once again, another application of absorption theorem

BC+A' - This is our final answer. Now take a look at the minimized circuit

In using boolean minimization, we are able to only use one and gate and one or gate. A reduction of 3 logic gates.

- Practice using Boolean Algebra.

Now you will create multiple circuits using the subsequent equations. On the surface, boolean minimization usually results in a more efficient circuit, but that isn’t always the case. Simplify the following equations.

2a. A'B'C'+A'B'C+A'C'

2b. AB'C+BD+A'B'C

- Understanding of prime implicants

When it comes to algebraic minimization, there are a lot of rules and theorems to memorize. To get around this, we use Kavanaugh Maps, K-Maps for short. We use these to find Implicants. An implicant can be defined as a product/minterm term in Sum of Products or sum/maxterm term in Product of Sums of a Boolean function.

ABC+A'+A'BC first we must find the minterms where  ∑f{0,1,2,3,7}

3a. Draw the K-Map of the equation, and find the simplified equation

3b. Simplify the following equation with a K-map. Is it the same as using boolean algrebra?  ABC+A'B+B'C