ECE 465 Lab 4

Encoders

- Introduction

The purpose of this lab is to provide hands on experience with encoders and decoders. You will create your own encoders in circuitverse and answer some questions.

- Encoders

An encoder is a digital combinational circuit that converts multiple input signals into a binary code. It typically has one active input at a time and generates a binary output representing the position of that active input. A typical encoder has 2n input lines with n output lines.

2a.  Below is the truth table for a 4 to 2 encoder.

| Input | Output |
| --- | --- |
| 0001 | 00 |
| 0010 | 01 |
| 0100 | 10 |
| 1000 | 11 |

Using the truth table, construct a 4 to 2 encoder in Circuitverse.

2b. Recall the equation for the input and outputs of an encoder. How many outputs would an 8-bit encoder have? And how would you design one? You may use 4 input Or gates.

- Priority Encoders

Sometimes, when there’s multiple inputs a general encoder might have issues with the output. To overcome this, we use priority encoders where the MSB is the only input the encoder is reading.

3a. Below is the truth table for a priority encoder. Notice the don’t care values.

| Input | Output |
| --- | --- |
| 0000 | XX0 |
| 0001 | 001 |
| 001X | 011 |
| 01XX | 101 |
| 1XXX | 111 |

Before creating the circuit, create a k-map for each output and derive a boolean expression.